TPx Communication’s Fraud Guidelines

Fraud Guidelines

There are several ways that telecommunication services may be fraudulently accessed, but it is possible to take steps to minimize fraud exposure on your network. While we at TPx hope this never happens to you, we have developed some simple tips to help you minimize your exposure to fraud and to help get to the bottom of the situation if you discover fraudulent activities.

You acknowledge that it is your responsibility to take whatever actions you deem necessary to make your computer and voice network and circuits adequately secure from unauthorized access. You further acknowledge that we only provide telecommunications services and certain equipment to you and that we are not responsible for the security of your network and circuits from third parties, or for any damages that may result from any unauthorized access to your network. We urge you to seek independent advice with respect to products, equipment (including configurations), and services available to make your computer network and circuits more secure from third parties.

If you suspect you may be the victim of fraud, call TPx’ s Fraud and Security Compliance Department at (866) 839-8545, Monday through Friday, from 7:30 AM to 5:00 PM PDT as soon as possible and notify your PBX and voicemail vendors.

The following are strongly recommended steps that, if taken, will help protect your network from hackers. Failure to implement these reasonable steps may expose your phone system to a greater likelihood of exposure to fraud. In addition, you should consult your telephone system vendor for additional guidance.

If the root cause of the fraud is in your phone equipment or from calls you allowed, your company will be held responsible for the fraudulent activity, including any charges that may be incurred.

A. Private Branch Exchange (PBX) Security

A PBX is a telephone system that serves a particular business or office. If the PBX system is not maintained and secured it can be compromised allowing remote hackers to originate long distance and international calls undetected to the business.

- Change the generic code (administration security code) and or existing code. A generic code is the default administration security code set within a PBX/phone system. This code allows administrators to control the “on and off” features which provides remote access within the PBX/phone systems. Obtaining generic codes for specific makes and models of PBX/phone systems are made available in the internet, therefore exposure exists, with hackers obtaining these codes.

- Understand the configuration of your PBX/phone systems and eliminate remote access and/ or disable access system. This is most important on IP PBXs in which SIP trunks and open ports for the Public Internet exist.

- Check for errors and changes in the configurations of your PBX, firewall and router or call processing,

- Install a firewall and anti-virus software.

- Check for inbound data spikes.

- Set up call accounting software or station message detail to monitor abnormal call activities. Set thresholds to alarm on abnormal call activities during business hours and off hours.

- Block all international countries to include Canada and Caribbean if not normally called directly on your PBX/phone system. A list of the international country codes is provided in this Fraud Guideline.

- If you do not accept third party billing, block this in your PBX/phone system.

- Block casual calling on your PBX/phone system. Casual calling allows callers to place long distance calls using another carrier identification code (CIC). The caller dials 1010, CIC and the destination number. The caller will receive a third-party bill from the third-party carrier. Fraudsters often use this method of dialing.

- Disable or change the password for Direct Inward System Access - DISA. This feature allows a caller to dial into the system, enter an authorization code and get an outbound line. The codes are often not difficult to crack. A hacker can then use this feature to make long distance calls at the company’s expense. The DISA application will provide dial tone.  If the password parameter is "no-password” the calls can complete.

- Add long distance or international account codes.

- Invest in a call accounting software.

- Run periodic security audits to check for vulnerabilities.

- Set a specific threshold for attempts to enter system and program the PBX to terminate access when threshold is exceeded.

- Discard listings and/ or directories with PBX access numbers by shredding the information.

- Share system information with only authorized individuals within the company.

- Review the features that are available on the system and disable those features that are not required.

- Disable, if possible, all forms of automated trunk to trunk (straight through dialing). Straight through dialing allows you to make telephone calls through your mailbox or telephone system when you are at an offsite location. If this feature is used, it is important that you generate and monitor reports to ensure your mailboxes are not being abused.

B. Voice Mail System

- Change the generic code (administration security code) and/ or existing code. A generic code is the default administration security code set within a voicemail system. This code allows administrators to have control of turning on and off features within the voice mail systems. Obtaining generic codes for specific makes and models of voice mail systems are made available in the internet, therefore exposure exist with hackers obtaining these codes.

- Set voicemail passwords to hard codes consisting of 6 characters to 10 characters. You must not use extensions and or soft codes 1111 and 1234. The more complex the security codes the harder it is to compromise.

- Change user security codes at least every 90 days.

- Understand the configuration of your voicemail system and disallow all features which allow remote access into your voicemail equipment to place outbound long distance and international calls. Hackers who have compromised a voicemail box will preprogram the fraud destination number(s). After they have done this, they use the transfer feature, pager feature and/or zero out features to place their fraud calls.

- Block 011 international outbound calls and calls to the Caribbean countries.

- Block 1010 casual calling and third party calls within your voicemail system.

- Disable unused voice mail boxes especially in unoccupied areas.

- Do not share your voice mail security codes or have them posted.

8YY (Toll Free) Robo Calls

8YY Robo Calls – There are many mixed abuse, targeted at 8yy services. The calls are mainly auto generated by a machine and produce large volumes in a short period of time, targeting toll free numbers. Long durations are often associated with these calls. In many cases, they will target an Interactive Voice Response (IVR) system, because calls can keep hitting prompts without dealing with a human that would hang up the call.

- Set up disconnect supervision on the IVR, auto attendant and voicemail. The caller should not be able to loop around in the IVR by pressing certain prompts. This is the key in avoiding long duration calls.

- Request to block certain area codes, from calling to your toll free number if you don’t have business needs.

VoIP SIP Security Recommendations

  - Do not accept SIP authentication requests from all IP addresses. Use the “permit=” and “deny=” lines in sip.conf to only allow a reasonable subset of IP addresses to reach each listed extension/user in your sip.conf file.  Even if you accept inbound calls from “anywhere” (via [default]) don’t let users reach authenticated elements.

  - Set “alwaysauthreject=yes” in your sip.conf file. The default is “no”, which allows extension information leakage.  Setting this to “yes” will reject bad authentication requests on valid usernames with the same rejection information as with invalid usernames, denying remote attackers the ability to detect existing extensions with brute-force guessing attacks.

  - Use STRONG passwords for SIP entities. Don’t just concatenate two words together and suffix it with “1″. Use symbols, numbers, and a mix of upper and lowercase letters at least 12 digits long.

  - Block your AMI manager ports. Use “permit=” and “deny=” lines in manager.conf to reduce inbound connections to known hosts only.  Use strong passwords, at least 12 characters with a complex mix of symbols, numbers, and letters.

  - Allow only one or two calls at a time per SIP entity, where possible. This limits your exposure when legitimate password holders on the system lose control of their passphrase.

  - Make your SIP usernames different from the extensions. While it is convenient to have extension “1234″ map to SIP entry “1234″ which is also SIP user “1234″, this is an easy target for attackers to guess SIP authentication names.  Use the MAC address of the device, or some sort of combination of a common phrase + extension MD5 hash (example: from a shell prompt, try “md5 -s ThePassword5000″)

  - Ensure your [default] context is secure.  Don’t allow unauthenticated callers to reach any contexts that allow toll calls.  Permit only a limited number of active calls through your default context (use the “GROUP” function as a counter.)  Prohibit unauthenticated calls entirely (if you don’t want them) by setting “allowguest=no” in the [general] part of sip.conf.

C. Physical Security

- Keep phone rooms secured and locked.

- Validate credentials for all technicians who visit your sites. Fraud perpetrators can gain access to unsecured phone rooms. A device can be clipped on to your line(s) to place fraud calls.

- Develop a security plan with your shared tenants and building management to secure your phone room.

D. Social Engineering

“Social Engineering” may be used, where a caller manipulates someone into performing an action or divulging confidential information. When the types of calls described below are received, report the call to TPx’s Fraud and Security Compliance Department at (866) 839-8545.

A fraud perpetrator may call into a business establishment pretending to be a technician for a phone company. The perpetrator will manipulate the party into pressing certain digits on the telephone keypad, allowing the perpetrator to place free long distance calls. The party will be charged for the long-distance calls.

Do not transfer callers to 900, 800, and 700. This is a fraud scam. Dialing 9, will provide the caller with an outside line and “00”, will send the caller to the long-distance operator. Any completed call placed will be charged to the business that transferred the caller to 900, 800, or 700.

E. Modem Hijacking

Modem Hijacking may be used to make fraudulent long distance calls from your computer that is billed to your phone number. Possible steps to help prevent modem hijacking include:

- Turn off your computer and modem when they are not in use.

- Unplug phone connections from the modem when a modem is not in use.

- Raise the volume level of your modem, so that you are aware of a redial.

- Be aware when visiting sites of questionable content, or avoid such sites entirely.  Several sites download surreptitiously onto consumers’ hard drives.

- Monitor your child’s use of the Internet.  Consider using blocking software to keep children from questionable sites – adult, gambling, etc.

- Do not download from an un-trusted web site.

- Close pop-up windows by selecting the, “X” button in the upper right hand corner, rather than any other embedded icons.

- Keep your operating system current with patches and updates.

- Ensure that Internet dial-up access numbers are on your local telephone plan and delete unknown access numbers.

- Consider installing anti-virus software and update regularly.

- Disable ActiveX.

- Install software to help prevent fraudulent activity.  Web sites where anti-virus or trial version of anti-virus software has been made available include:

  - www.symantec.com

  - www.mcafee.com

F. International Country Codes List to Block Directly within the PBX.

International Country Code List

Code	Country

20	Egypt

211	South Sudan

212	Morocco

213	Algeria

216	Tunisia

218	Libya

220	Gambia

221	Senegal

222	Mauritania

223	Mali

224	Guinea

225	Ivory Coast

226	Burkina Faso

227	Niger

228	Togo

229	Benin

230	Mauritius

231	Liberia

232	Sierra Leone

233	Ghana

234	Nigeria

235	Chad

236	Central African Republic

237	Cameroon

238	Cape Verde

239	São Tome

240	Equatorial Guinea and Principe

241	Gabon

242	Republic of the Congo

243	Democratic Republic of the Congo

244	Angola

245	Guinea-Bissau

246	British Indian Ocean Territory

247	Ascension Island

248	Seychelles

249	Sudan

250	Rwanda

251	Ethiopia

252	Somalia

253	Djibouti

254	Kenya

255	Tanzania

256	Uganda

257	Burundi

258	Mozambique

260	Zambia

261	Madagascar

262	Réunion

263	Zimbabwe

264	Namibia

265	Malawi

266	Lesotho

267	Botswana

268	Swaziland

269	Comoros

27	South Africa

290	Saint Helena

291	Eritrea

297	Aruba

298	Faroe Islands

299	Greenland

30	Greece

31	Netherlands

32	Belgium

33	France

34	Spain

350	Gibraltar

351	Portugal

352	Luxembourg

353	Ireland

354	Iceland

355	Albania

356	Malta

357	Cyprus

358	Finland

359	Bulgaria

36	Hungary

370	Lithuania

371	Latvia

372	Estonia

373	Moldova

374	Armenia

375	Belarus

376	Andorra

377	Monaco

378	San Marino

379	Vatican City

380	Ukraine

381	Serbia

382	Montenegro

383	Kosovo

385	Croatia

386	Slovenia

387	Bosnia and Herzegovina

388	Discontinued

389	Macedonia

39	Italy

40	Romania

41	Switzerland

420	Czech Republic

421	Slovakia

422	unassigned

423	Liechtenstein

43	Austria

44	United Kingdom

45	Denmark

46	Sweden

47	Norway

48	Poland

49	Germany

500	Falkland Islands

501	Belize

502	Guatemala

503	El Salvador

504	Honduras

505	Nicaragua

506	Costa Rica

507	Panama

508	Saint-Pierre and Miquelon

509	Haiti

51	Peru

52	Mexico

53	Cuba

54	Argentina

55	Brazil

56	Chile

57	Colombia

58	Venezuela

590	Guadeloupe

591	Bolivia

592	Guyana

593	Ecuador

594	French Guiana

595	Paraguay

596	Martinique

597	Suriname

598	Uruguay

599	Netherlands Antilles

60	Malaysia

61	Australia

62	Indonesia

63	Philippines

64	New Zealand

65	Singapore

66	Thailand

670	East Timor

672	Australian External Territories

673	Brunei

674	Nauru

675	Papua New Guinea

676	Tonga

677	Solomon Islands

678	Vanuatu

679	Fiji

680	Palau

681	Wallis and Futuna

682	Cook Islands

683	Niue

685	Samoa

686	Kiribati

687	New Caledonia

688	Tuvalu

689	French Polynesia

690	Tokelau

691	Federated States of Micronesia

692	Marshall Islands

7	Russia/Kazakhstan/Abkhazia

81	Japan

82	South Korea

84	Vietnam

850	North Korea

852	Hong Kong

853	Macau

855	Cambodia

856	Laos

86	China

870	Inmarsat SNAC Service

878	Universal Personal Telecommunications Services

879	reserved

880	Bangladesh

881	Global Mobile Satellite System

882	International Networks

883	International Networks

886	Taiwan

888	Telecommunications for Disaster Relief by OCHA

889	unassigned

90	Turkey

91	India

92	Pakistan

93	Afghanistan

94	Sri Lanka

95	Myanmar

960	Maldives

961	Lebanon

962	Jordan

963	Syria

964	Iraq

965	Kuwait

966	Saudi Arabia

967	Yemen

968	Oman

970	Palestine

971	United Arab Emirates

972	Israel

973	Bahrain

974	Qatar

975	Bhutan

976	Mongolia

977	Nepal

979	International Premium Rate Service

98	Iran

991	International Telecommunications Public Correspondence Service

992	Tajikistan

993	Turkmenistan

994	Azerbaijan

995	Georgia

996	Kyrgyzstan

998	Uzbekistan

Caribbean Dialing Code List (1+)

1+	340	United States Virgin Islands

1+	670	Northern Mariana Islands

1+	671	Guam

1+	684	American Samoa

1+	787	Puerto Rico

1+	242	Bahamas

1+	246	Barbados

1+	264	Anguilla

1+	268	Antigua and Barbuda

1+	284	British Virgin Islands

1+	345	Cayman Islands

1+	441	Bermuda

1+	473	Grenada

1+	649	Turks and Caicos Islands

1+	664	Montserrat

1+	721	Sint Maarten

1+	758	Saint Lucia

1+	767	Dominica

1+	784	Saint Vincent and the Grenadines

1+	809	Dominican Republic

1+	829	Dominican Republic

1+	849	Dominican Republic

1+	868	Trinidad and Tobago

1+	869	Saint Kitts and Nevis

1+	876	Jamaica

1+	939	Puerto Rico